HLS Pragma Analysis Report

Hybrid  |  SHA-3  |  Ascon

This report documents every #pragma HLS directive present in the three implementations, explains the reasoning behind each one, and identifies errors and possible improvements. The document is divided into three parts: Hybrid, SHA-3, and Ascon.

| Status | Meaning |
| --- | --- |
| OK | Pragma is correct and effective |
| OK — ignored | Pragma is present but has no effect in the current configuration |
| ERROR | Pragma is missing, misplaced, or counterproductive |

# Part 1 — Hybrid Implementation

The hybrid file routes both SHA-3 and Ascon through the single function keccakf_asconp12(), selected at runtime by a mode parameter (0 = Ascon, >0 = SHA-3). All loop bounds that depend on mode are therefore runtime variables, which limits what the HLS tool can do statically.

## keccakf_asconp12()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| keccakf_asconp12 | #pragma HLS inline off | Prevents inlining at every call site. Without this, the full permutation would be duplicated at each call in init, update, final, and the squeezing loops, causing a large area explosion. | OK |
| rounds_loop | // #pragma HLS pipeline II=1 | Commented out. nrounds is derived from the runtime mode variable, so the tool cannot determine the trip count statically and the pragma would be ignored. Correctly omitted. | OK |

## theta()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| theta | #pragma HLS INLINE | Inlines theta() into its caller so the tool can optimise it as part of the larger pipeline region. | OK |
| theta / bc[] | // #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | Commented out. Without this, bc[] lives in a single-port block RAM and all loop iterations are serialised. This is the most impactful missing pragma in theta() — it prevents II=1 on every loop inside the function. | ERROR — missing |
| theta_C loop | #pragma HLS PIPELINE II=1 | Requests II=1 on the column-parity loop. Depends on bc[] being partitioned; without it the single-port memory bottleneck forces II > 1. | OK — ignored (bc[] not partitioned) |
| theta_D loop | #pragma HLS PIPELINE II=1 | Same rationale as theta_C. | OK — ignored (bc[] not partitioned) |
| theta_D_loop (innermost) | #pragma HLS PIPELINE II=1 | Applied to the inner j-loop while the outer theta_D loop already has a PIPELINE pragma. HLS tools do not support nested pipeline directives on directly nested loops — the inner one is discarded and may confuse the scheduler. | ERROR — nested PIPELINE conflict |

## rho_pi()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| rho_pi | #pragma HLS INLINE | Inlines the function into the round region. Required for the outer pipeline to absorb it. | OK |
| rho_pi / bc[] | // #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | bc[] is used only as a one-element scratch (bc[0]). Partitioning it has no practical benefit. Harmless to leave commented out. | OK — low impact |
| rho_pi_loop | #pragma HLS PIPELINE II=1 | The loop carries a dependency on t through every iteration. This is a true loop-carried dependency that may prevent the tool from reaching II=1. | OK — may not reach II=1 |

## chi()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| chi | #pragma HLS INLINE | Inlines chi() into the round pipeline. | OK |
| chi / bc[] | // #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | Commented out. bc[] is read across all five iterations of chi_nlin. Without partitioning, those reads are serialised, making it impossible to achieve II=1 on the nonlinear loop. Most impactful missing pragma in chi(). | ERROR — most impactful missing pragma |
| top_chi loop | // #pragma HLS pipeline II=1 | Commented out. chirounds is a runtime variable in the hybrid, so the outer Chi loop cannot be statically unrolled. Without a pipeline pragma here there is no loop-level parallelism on the outer loop at all. | ERROR — should be enabled |
| bc_cp_chi loop | #pragma HLS PIPELINE II=1 | Pipelines the copy into bc[]. Ineffective without bc[] partitioned. | OK — ignored (bc[] not partitioned) |
| chi_nlin loop | #pragma HLS PIPELINE II=1 | Pipelines the nonlinear computation. Same constraint. | OK — ignored (bc[] not partitioned) |

## iota()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| iota | #pragma HLS INLINE | Single XOR. Inlining is correct and the tool would likely do it automatically. | OK |

diffusion()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| diffusion | #pragma HLS INLINE | Inlines the function into the round pipeline. | OK |
| diffusion / bc[] | // #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | Commented out. bc[] holds the five Ascon words used for the rotation operations. Partitioning allows the copy loop and the linear assignments to overlap in the pipeline schedule. | ERROR — should be enabled |
| bc_cp_diff loop | #pragma HLS PIPELINE II=1 | Pipelines the 5-iteration copy loop. Only effective with bc[] partitioned. | OK — ignored (bc[] not partitioned) |

## hash_init() / hash_update() / hash_final()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| init_loop | #pragma HLS PIPELINE II=1 | Pipelines the 25-word state zeroing loop. Straightforward and effective. | OK |
| absorbing_loop | no pipeline pragma | Contains a conditional call to keccakf_asconp12(). Pipelining a loop with an internal function call forces HLS to inline the entire permutation into the loop body, causing a resource explosion. Correctly left unpipelined. | OK — intentionally absent |
| output_loop (SHA-3 final) | // pipeline (commented out) | mdlen is a runtime variable — the tool cannot determine the trip count. Correctly left without a pipeline pragma. A LOOP_TRIPCOUNT hint could help latency estimates. | OK |
| squeezing loops (Ascon final) | // pipeline (commented out) | Four fixed 8-iteration loops with no function calls inside. Safe to pipeline or unroll for a small latency saving during the squeeze phase. | OK — minor improvement possible |

## Summary — Errors and Improvements

| ERROR | bc[] array partitioning is commented out in theta(), chi(), and diffusion(). This single missing pragma is responsible for the largest performance gap: all pipeline pragmas on the inner loops are ineffective because array accesses are serialised through a single memory port. |
| --- | --- |
| ERROR | The top_chi pipeline pragma is commented out in the hybrid. Since chirounds is a runtime variable (5 or 25), the outer Chi loop cannot be statically unrolled and the pipeline pragma is needed to achieve any loop-level parallelism. |
| ERROR | theta_D_loop (innermost) has its own PIPELINE pragma alongside the outer theta_D PIPELINE. Nested pipeline directives on directly nested loops are not supported — the inner one is discarded and may confuse the scheduler. |
| Improvement | The four squeezing loops in hash_final() (Ascon branch) each iterate exactly 8 times with no function calls. Adding UNROLL or PIPELINE II=1 to each would eliminate loop overhead for a free latency reduction. |
| Improvement | The mode branching inside keccakf_asconp12() prevents the tool from specialising the permutation. Splitting into two dedicated functions would let the tool fully optimise each path independently and remove the runtime branch overhead. |

# Part 2 — SHA-3 Only Implementation

This version removes the mode parameter and is dedicated to Keccak/SHA-3. All loop bounds are compile-time constants (24 rounds, chirounds = 25), giving the tool full visibility for unrolling and scheduling.

## keccakf_asconp12()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| keccakf_asconp12 | #pragma HLS inline off | Same rationale as the hybrid — prevents duplication at multiple call sites. | OK |
| rounds_loop | #pragma HLS pipeline II=1 | nrounds = 24 is now a compile-time constant. Pipelining the round loop at II=1 maximises throughput at the cost of significant area. | OK |

## theta()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| theta | #pragma HLS INLINE | Required for the round pipeline to absorb the full round logic. | OK |
| theta / bc[] | #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | Now enabled (not commented out). Splits bc[] into 5 independent registers, enabling the inner loops to be fully unrolled and the 5 XOR operations to execute in parallel. Key enabler for II=1 on the theta loops. | OK |
| theta_C / theta_D loops | #pragma HLS PIPELINE II=1 | Now effective because bc[] is partitioned. | OK |
| theta_D_loop (innermost) | #pragma HLS PIPELINE II=1 | Same nested-pipeline conflict as in the hybrid. The outer theta_D loop already has PIPELINE; the inner directive on the j-loop is discarded. | ERROR — redundant, remove |

## rho_pi()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| rho_pi | #pragma HLS INLINE | Correct. | OK |
| rho_pi / bc[] | #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | Present. bc[] is used only as a one-element scratch (bc[0]), so the partition has no practical effect. Harmless. | OK — no effect |
| rho_pi_loop | #pragma HLS PIPELINE II=1 | Loop-carried dependency on t may prevent II=1. The tool may settle at II=2 or higher. | OK — may not reach II=1 |

## chi()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| chi | #pragma HLS INLINE | Correct. | OK |
| chi / bc[] | #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | Present. Enables all 5 bc[] reads in chi_nlin to happen in parallel — most important pragma inside chi(). | OK |
| top_chi loop | #pragma HLS pipeline II=1 | chirounds = 25 is a constant (5 outer iterations). Pipeline controls area vs. throughput. Acceptable. | OK |
| bc_cp_chi / chi_nlin | #pragma HLS PIPELINE II=1 | Both inner loops benefit from bc[] partitioning and are now effective. | OK |

## iota()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| iota | #pragma HLS INLINE | Correct. | OK |

## hash_init() / hash_update() / hash_final()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| init_loop | #pragma HLS PIPELINE II=1 | Effective. | OK |
| absorbing_loop | no pipeline pragma | Correctly absent. | OK — intentionally absent |
| output_loop | #pragma HLS pipeline II=1 | Added in this version. mdlen is still a runtime variable, so the tool may not honour the pragma without a LOOP_TRIPCOUNT hint. | OK — add LOOP_TRIPCOUNT |

## Summary — Errors and Improvements

| ERROR | theta_D_loop (innermost) still carries a PIPELINE pragma that conflicts with the outer theta_D PIPELINE. Inherited from the hybrid — remove the inner one. |
| --- | --- |
| Improvement | The output_loop pipeline pragma is present but mdlen is a runtime variable. Adding #pragma HLS LOOP_TRIPCOUNT min=32 max=64 would allow the tool to produce accurate latency estimates even without static unrolling. |
| Improvement | With all bounds now compile-time constants, replacing PIPELINE II=1 on rounds_loop with UNROLL would fully unroll all 24 rounds into combinational logic for maximum throughput at higher area cost. |

# Part 3 — Ascon Only Implementation

This version implements only Ascon-p12: 12 rounds, 5-word state, chirounds = 5. All loop bounds are compile-time constants. The Keccak-specific steps (theta, rho_pi, iota) are absent.

## keccakf_asconp12()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| keccakf_asconp12 | #pragma HLS inline off | Prevents inlining across multiple call sites. Correct. | OK |
| rounds_loop | #pragma HLS pipeline II=1 | nrounds = 12 is a constant. With all sub-functions inlined, the tool can pipeline the full round body. Correct and expected. | OK |

## chi()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| chi | #pragma HLS INLINE | Inlines chi() into the round pipeline. | OK |
| chi / bc[] | #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | chirounds = 5 means the outer loop has one iteration, which the tool may unroll automatically, but the inner bc_cp_chi and chi_nlin loops still need bc[] partitioned to run in parallel. | OK |
| top_chi loop | #pragma HLS pipeline II=1 | chirounds = 5 means the outer loop executes exactly once (j = 0 only). A PIPELINE on a single-iteration loop adds scheduler overhead with no throughput benefit. UNROLL would be more appropriate. | OK — UNROLL preferred |
| bc_cp_chi / chi_nlin | #pragma HLS PIPELINE II=1 | Both inner loops have constant 5-iteration bounds. Effective once bc[] is partitioned; currently ignored. | OK |

## diffusion()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| diffusion | #pragma HLS INLINE | Inlines the function into the round pipeline. | OK |
| diffusion / bc[] | #pragma HLS ARRAY_PARTITION variable=bc complete dim=1 | Partitioning would allow the copy loop and the subsequent independent scalar assignments to be scheduled in fewer pipeline stages. | OK |
| bc_cp_diff loop | #pragma HLS PIPELINE II=1 | 5-iteration copy loop. Effective with bc[] partitioned; serialised without it. | OK |

## hash_init() / hash_update() / hash_final()

| Location | Pragma | Purpose | Status |
| --- | --- | --- | --- |
| init_loop | #pragma HLS PIPELINE II=1 | Effective. | OK |
| absorbing_loop | no pipeline pragma | Correctly absent — contains a conditional permutation call. | OK — intentionally absent |
| squeezing_1 to squeezing_4 | // pipeline / unroll (commented out) | Four fixed 8-iteration loops with no function calls. Safe to add UNROLL for a small, free reduction in squeeze latency. | OK — improvement possible |

## Summary — Errors and Improvements

| ERROR | top_chi uses PIPELINE when chirounds = 5 produces only one outer iteration (j = 0). A pipeline on a single-trip loop is wasteful — replace with #pragma HLS UNROLL. |
| --- | --- |
| Improvement | The four squeezing loops in hash_final() each have exactly 8 iterations and no function calls. Adding #pragma HLS UNROLL to each eliminates all loop overhead during the output phase at negligible area cost. |
| Improvement | The init_loop zeros all 25 state words, but Ascon uses only 5 of them. Reducing the loop bound to 5 saves 20 unnecessary write cycles with no correctness impact. |
| Improvement | With nrounds = 12 and all sub-functions inlined, replacing rounds_loop PIPELINE with UNROLL would fully unroll all 12 rounds into combinational logic for maximum throughput — useful if latency is the primary constraint. |

End of Report